Request Title:  	Request to use QDPR(Quantum Dots Photo Resist)

Requestor: 		Tracy Lu			Date:  June 18th , 2020

Non-SNF staff, please complete the following:

Badger ID:	tracylu@royole.com

Company name/PI: Royole Corporation

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

The name of this Quantum Dots Photo Resist is “QDPR ver.6 series(R&D)”. This QDPR is not commercial product yet. It’s designed for research and development use only.

Below are the constituents of the chemical and their corresponding CAS numbers.

QDPR ver.6 series(R&D) hazardous ingredients:

| Component | CAS-No. | Weight percent |
| --- | --- | --- |
| Propylene glycol monomethyl ether acetate (PGMEA) | 108-65-6 | 40~80% |
| Semiconductor nanoparticles | Trade secret | 5~20% |
| Binder | Trade secret | 5~30% |
| Monomer | Trade secret | 1-20% |
| Antioxidant | Trade secret | <2% |
| Scattering Agent | Trade secret | 0.5~10% |
| Photoinitiator | Trade secret | <3% |
| Additives | Trade secret | <1% |

Please find attached the MSDS for the chemical in the same folder as this document.

- Vendor/manufacturer info: address and phone number, website URL.

| Company Name: | Sumitomo Chemical Co., Ltd. |
| --- | --- |
| Address: | 1-98, Kasugade-naka 3-chome, Konohana-ku, Osaka, 554-8558 Japan |
| Department: | IT-related Chemicals Research Laboratory |
| Phone Number: | +81-6-6466-5811 |
| FAX Number: | +81-6-6466-5489 |

Website:            https://www.sumitomo-chem.co.jp/english/

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

This quantum dots photo resist will be applied and fabricated in our devices for the display applications. QDPR is a relatively new technology in display and there is no substitute in SNF for our requirement.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

Below are the proposed processing steps:

- Spin coat the resist on Laurell-G.

- Soft-bake using hotplate at 100C for 3mins.

- Expose using Karlsuss Contact Aligner or Heidelburg MLA.

- Develop using MF-26A and rinse with DI water on the Lithosolv Bench. (vendor recommends 0.048% KOH-aq for development; If MF-26A doesn’t work well, I will send out a new PROM request for discussion of carrying out KOH for development)

- Hard-bake using White Oven or Bluem Oven at 180C for 60mins.

All excess solvent and resist will be poured into the solvent waste carboys and wipes, foils, waste swabs etc. will be placed into zip-locks and disposed-off into the solvent trash can.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

We will store one bottle of 60g of red QDPR and one bottle of 60g green QDPR in the refrigerator at SNF.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

We propose to store the resist in the freezer inside the SNF cleanroom, where all the other resists, spin-on-glass, etc. are stored.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

We will dispose-off excess resist into the solvent waste carboys or seal in plastic zip lock bags and dispose-off in the solvent waste receptacles.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: